TNC-2 1.1.8 QRA Mode

QRA  ("Who  are  you")  pinging is supported in release 1.1.8 and

clone  equivalents.  A  QRA  ping  polls  all  of the TNCs within

range.  A random number of seconds later, each TNC that heard the

QRA ping will transmit its identification packet.  The purpose of

this  feature  is  to  reveal  to the packeteer, particularly the

transient one, the names of all reachable TNCs and digipeaters.

Sending a QRA ping:

1) Set the unprotocol callsign to QRA

cmd: UNPROTO  QRA<cr>

2) Manually send an unconnected packet

cmd: CONVERSE<cr>

<cr>

^C

cmd:

new command:

ANswrqra  ON|OFF                             default: ON

Setting  ANSWRQRA  to  OFF disables the TNC's ping-response func-

tion.  When  the  parameter  is  set ON, the TNC responds to non-

digipeated  UI  frames  addressed to QRA, within 1 to 16 seconds,

with an empty ID packet.

#############################################################################