MFJ1278 UIDIGI modification for like TNC2 compatible to work with UIDIGI

Author: Tim Cunningham – N8DEU

Location: Huntsville, Alabama, USA

Construct a header to replace U27

Radio Port 1; VHF Packet;

Connect pins 2, 5, and 15 to pin 20 (+5V)

Connect pins 9, 12, 16, and 19 to pin 10 (GND)

Construct a header to replace U28

2400 baud terminal; 1200 baud radio

Connect pins 2,5,9,12,15, and 19 to pin 10 (GND)

Connect pins 6 and 16 to pin 20 (+5V)

Remove the following IC’s from the unit:

U29, U37, U38, and U39

Notes:

U27 and U28 must be removed and replaced with the headers that have been constructed. The MFJ-1278 is not a TNC2 compatible unit in its native state. Therefore, you must modify the unit to make it a TNC2 compatible. Normally, the original firmware in the MFJ1278 controls the baud rate of the serial port and the radio modem through U27 and U28. Since the UIDIGI firmware does not control this function you must physically hard-wire these settings by replacing U27 and U28 with headers wired to program the radio port to 1200 baud and the serial port to 2400 baud. The serial port was configured for 2400 baud to support many of our weather station interfaces. If you need a different serial port setting you will need to modify the wiring on the U28 header connector.

With the changes implemented with the headers at U27 and U28 the unit will work with the original 1278 firmware at 2400 baud 7E1 but not with Pac-Comm or UIDIGI firmware installed. The characters on the serial port do not look correct with Pac-Comm or UIDIGI. Keep in mind the radio port and serial port settings are hard-wired and the 1278 firmware can no longer control these settings within the firmware.

The initial sign-on message will display "-8 1o3" or "-8 g3". Apparently some characters were just being skipped. After some probing with an oscilloscope and some experimenting, I found that removing U29, U37, U38, and U39 solved the problem. More specifically just the removal of U38 seemed to fix this problem.

In any case, the removal of these chips reduces the power consumption and removes the possibility that these chips would cause any failures in the future. If any of the unused chips are left in the unit and one of them fails, it seems silly to allow this type of potential failure when the chips are unused.

There is a simpler modification that requires less wiring and the removal of two additional IC’s, but I need to retrieve the unit I modified several years ago in order to verify that simple modification before I document it.

The modification covered in this document has allowed us to utilize MFJ-1278 TNC’s in our UIDIGI network.